Preservation of Mutants

. Thus, preservation techniques have been developed to maintain cultures in a state of 'suspended animation' The isolation of a suitable organism for a commercial process may be a long and very expensive procedure and it is therefore essential that it retains the desirable characteristics that led to its selection. Also, the culture used to initiate an industrial fermentation must be viable and free from contamination. Thus, industrial cultures must be stored in such way as to eliminate genetic change, protect against contamination and retain viability. An organism may be kept viable by repeated sub-culture into fresh medium, but, at each cell division, there is a small probability of mutations occurring and because repeated sub-culture involves very many such divisions, there is a high probability that strain degeneration would occur. Also, repeated sub-culture carries with it the risk of contamination by storing either at reduced temperature or in a dehydrated form.

Storage at reduced temperature

STORAGE ON AGAR SLOPES

Cultures grown on agar slopes may be stored in a refrigerator (5°) or a freezer (- 20°) and subcultured at approximately 6-monthly intervals. The time of subculture may be extended to I year if the slopes are covered with sterile medicinal grade mineral oil.

STORAGE UNDER LIQUID NITROGEN

The metabolic activities of micro-organisms may be reduced considerably by storage at the very low temperatures (-150° to -196°) which may be achieved using a liquid nitrogen refrigerator. Snell (1991) claimed that this aproach is the most universally applicable of all preservation methods. Fungi, bacteriophage, viruses, algae, yeasts, animal and plant cells and tissue cultures have all been successfully preserved. The technique involves growing a culture to the maximum stationary phase, resuspending the cells in a cryoprotective agent (such as 10% glycerol) and freezing the suspension in sealed ampoules before storage under liquid nitrogen. Some loss of viability is suffered during the freezing and thawing stages but there is virtually no loss during the storage period.

Storage in a dehydrated form

DRIED CULTURES

Dried soil cultures have been used widely for culture preservation, particularly for sporulating mycelial organisms.

Moist, sterile soil may be inoculated with a culture and incubated for several days for some growth to occur and then allowed to dry at room temperature for approximately 2 weeks. The dry soil may be stored in a dry atmosphere or, preferably, in a refrigerator. The technique has been used extensively for the storage of fungi and actinomycetes that of 1800 actinomycetes dried on soil about 50% were viable after 20-years storage.

LYOPHILIZATION

Lyophilization, or freeze-drying, involves the freezing of a culture followed by its drying under vacuum, esults in the sublimation of the cell water. The technique involves growing the culture to the maximum stationary phase and resuspending the cells in a protective medium such as milk, serum or sodium glutamate. A few drops of the suspension are transferred to an

ampoule, which is then frozen and subjected to a high vacuum until sublimation is complete, after which the ampoule is sealed. The ampoules may be stored in a refrigerator and the cells may remain viable for 10 years or more.