Waste management is all the activities and actions required to manage waste from its inception to its final disposal.[1] This includes amongst other things, collection, transport, treatment and disposal of waste together with monitoring and regulation. It also encompasses the legal and regulatory framework that relates to waste management encompassing guidance on recycling etc.

The term usually relates to all kinds of waste, whether generated during the extraction of raw materials, the processing of raw materials into intermediate and final products, the consumption of final products, or other human activities,[1] including municipal (residential, institutional, commercial), agricultural, and social (health care, household hazardous wastes, sewage sludge).[2] Waste management is intended to reduce adverse effects of waste on health, the environment or aesthetics.

Waste management practices are not uniform among countries (developed and developing nations); regions (urban and rural area), and sectors (residential and industrial).[3]

There are a number of concepts about waste management which vary in their usage between countries or regions. Some of the most general, widely used concepts include:

### Waste hierarchy[edit]

The waste hierarchy refers to the "3 Rs" reduce, reuse and recycle, which classify waste management strategies according to their desirability in terms of waste minimisation. The waste hierarchy remains the cornerstone of most waste minimisation strategies. The aim of the waste hierarchy is to extract the maximum practical benefits from products and to generate the minimum amount of waste; see: resource recovery.The waste hierarchy is represented as a pyramid because the basic premise is for policy to take action first and prevent the generation of waste. The next step or preferred action is to reduce the generation of waste i.e. by re-use. The next is recycling which would include composting. Following this step is material recovery and waste-to-energy. Energy can be recovered from processes i.e. landfill and combustion, at this level of the hierarchy. The final action is disposal, in landfills or through incineration without energy recovery. This last step is the final resort for waste which has not been prevented, diverted or recovered.[4] The waste hierarchy represents the progression of a product or material through the sequential stages of the pyramid of waste management. The hierarchy represents the latter parts of the life-cycle for each product.[4]

### Life-cycle of a product[edit]

The life-cycle begins with design, then proceeds through manufacture, distribution, use and then follows through the waste hierarchy's stages of reuse, recovery, recycling and disposal. Each of the above stages of the life-cycle offers opportunities for policy intervention, to rethink the need for the product, to redesign to minimize waste potential, to extend its use.[4] The key behind the life-cycle of a product is to optimize the use of the world's limited resources by avoiding the unnecessary generation of waste.

### Resource efficiency[edit]

Resource efficiency reflects the understanding that current, global, economic growth and development can not be sustained with the current production and consumption patterns. Globally, we are extracting more resources to produce goods than the planet can replenish.[4] Resource efficiency is the reduction of the environmental impact from the production and consumption of these goods, from final raw material extraction to last use and disposal. This process of resource efficiency can address sustainability.

### Polluter pays principle[edit]

The Polluter pays principle is a principle where the polluting party pays for the impact caused to the environment. With respect to waste management, this generally refers to the requirement for a waste generator to pay for appropriate disposal of the unrecoverable material.

Waste handling and transport

Waste collection methods vary widely among different countries and regions. Domestic waste collection services are often provided by local government authorities, or by private companies for industrial and commercial waste. Some areas, especially those in less developed countries, do not have formal waste-collection systems.

### Waste handling practices

Curbside collection is the most common method of disposal in most European countries, Canada, New Zealand and many other parts of the developed world in which waste is collected at regular intervals by specialised trucks. This is often associated with curb-side waste segregation. In rural areas waste may need to be taken to a transfer station. Waste collected is then transported to an appropriate disposal facility. In some areas, vacuum collection is used in which waste is transported from the home or commercial premises by vacuum along small bore tubes. Systems are in use in Europe and North America.

Pyrolysis is used to dispose of some wastes including tires, a process that can produce recovered fuels, steel and heat. In some cases tires can provide the feedstock for cement manufacture. Such systems are used in USA, California, Australia, Greece, Mexico, the United Kingdom and in Israel. The RESEM pyrolysis plant that has been operational at Texas USA since December 2011, and processes up to 60 tons per day.[12] In some jurisdictions unsegregated waste is collected at the curb-side or from waste transfer stations and then sorted into recyclables and unusable waste. Such systems are capable of sorting large volumes of solid waste, salvaging recyclables, and turning the rest into bio-gas and soil conditioner. In San Francisco, the local government established its Mandatory Recycling and Composting Ordinance in support of its goal of zero waste by 2020, requiring everyone in the city to keep recyclables and compostables out of the landfill. The three streams are collected with the curbside "Fantastic 3" bin system - blue for recyclables, green for compostables, and black for landfill-bound materials - provided to residents and businesses and serviced by San Francisco's sole refuse hauler, Recology. The City's "Pay-As-You-Throw" system charges customers by the volume of landfill-bound materials, which provides a financial incentive to separate recyclables and compostables from other discards. The City's Department of the Environment's Zero Waste Program has led the City to achieve 80% diversion, the highest diversion rate in North America.[13]

Disposal solutions

Landfill- A landfill site (also known as a tip, dump, rubbish dump, garbage dump or dumping ground and historically as a midden[1]) is a site for the disposal of waste materials by burial and is the oldest form of waste treatment (although the burial part is modern; historically, refuse was just left in piles or thrown into pits). Historically, landfills have been the most common method of organized waste disposal and remain so in many places around the world.

Some landfills are also used for waste management purposes, such as the temporary storage, consolidation and transfer, or processing of waste material (sorting, treatment, or recycling).

A landfill also may refer to ground that has been filled in with rocks instead of waste materials, so that it can be used for a specific purpose, such as for building houses. Unless they are stabilized, these areas may experience severe shaking or liquefaction of the ground during a large earthquake.

### Incineration-Incineration is a disposal method in which solid organic wastes are subjected to combustion so as to convert them into residue and gaseous products. This method is useful for disposal of residue of both solid waste management and solid residue from waste water management. This process reduces the volumes of solid waste to 20 to 30 percent of the original volume. Incineration and other high temperature waste treatment systems are sometimes described as "thermal treatment". Incinerators convert waste materials into heat, gas, steam, and ash.

Incineration is carried out both on a small scale by individuals and on a large scale by industry. It is used to dispose of solid, liquid and gaseous waste. It is recognized as a practical method of disposing of certain hazardous waste materials (such as biological medical waste). Incineration is a controversial method of waste disposal, due to issues such as emission of gaseous pollutants.

Incineration is common in countries such as Japan where land is more scarce, as these facilities generally do not require as much area as landfills. Waste-to-energy (WtE) or energy-from-waste (EfW) are broad terms for facilities that burn waste in a furnace or boiler to generate heat, steam or electricity. Combustion in an incinerator is not always perfect and there have been concerns about pollutants in gaseous emissions from incinerator stacks. Particular concern has focused on some very persistent organic compounds such as dioxins, furans, and PAHs, which may be created and which may have serious environmental consequences.

Recycling -Recycling is a resource recovery practice that refers to the collection and reuse of waste materials such as empty beverage containers. The materials from which the items are made can be reprocessed into new products. Material for recycling may be collected separately from general waste using dedicated bins and collection vehicles, a procedure called kerbside collection. In some communities, the owner of the waste is required to separate the materials into different bins (e.g. for paper, plastics, metals) prior to its collection. In other communities, all recyclable materials are placed in a single bin for collection, and the sorting is handled later at a central facility. The latter method is known as "single-stream recycling."[14][15]

The most common consumer products recycled include aluminium such as beverages cans, copper such as wire, steel from food and aerosol cans, old steel furnishings or equipment, rubber tyres, polyethylene and PET bottles, glass bottles and jars, paperboard cartons, newspapers, magazines and light paper, and corrugated fiberboard boxes.

PVC, LDPE, PP, and PS (see resin identification code) are also recyclable. These items are usually composed of a single type of material, making them relatively easy to recycle into new products. The recycling of complex products (such as computers and electronic equipment) is more difficult, due to the additional dismantling and separation required.

The type of material accepted for recycling varies by city and country. Each city and country has different recycling programs in place that can handle the various types of recyclable materials. However, certain variation in acceptance is reflected in the resale value of the material once it is reprocessed.

### Re-use

### Biological reprocessing-An active compost heap. Recoverable materials that are organic in nature, such as plant material, food scraps, and paper products, can be recovered through composting and digestion processes to decompose the organic matter. The resulting organic material is then recycled as mulch or compost for agricultural or landscaping purposes. In addition, waste gas from the process (such as methane) can be captured and used for generating electricity and heat (CHP/cogeneration) maximising efficiencies. The intention of biological processing in waste management is to control and accelerate the natural process of decomposition of organic matter. (See resource recovery).

### Energy recovery[edit]-Energy recovery from waste is the conversion of non-recyclable waste materials into usable heat, electricity, or fuel through a variety of processes, including combustion, gasification, pyrolyzation, anaerobic digestion, and landfill gas recovery.[16] This process is often called waste-to-energy. Energy recovery from waste is part of the non-hazardous waste management hierarchy. Using energy recovery to convert non-recyclable waste materials into electricity and heat, generates a renewable energy source and can reduce carbon emissions by offsetting the need for energy from fossil sources as well as reduce methane generation from landfills.[16] Globally, waste-to-energy accounts for 16% of waste management.[17]

The energy content of waste products can be harnessed directly by using them as a direct combustion fuel, or indirectly by processing them into another type of fuel. Thermal treatment ranges from using waste as a fuel source for cooking or heating and the use of the gas fuel (see above), to fuel for boilers to generate steam and electricity in a turbine. Pyrolysis and gasification are two related forms of thermal treatment where waste materials are heated to high temperatures with limited oxygen availability. The process usually occurs in a sealed vessel under high pressure. Pyrolysis of solid waste converts the material into solid, liquid and gas products. The liquid and gas can be burnt to produce energy or refined into other chemical products (chemical refinery). The solid residue (char) can be further refined into products such as activated carbon. Gasification and advanced Plasma arc gasification are used to convert organic materials directly into a synthetic gas (syngas) composed of carbon monoxide and hydrogen. The gas is then burnt to produce electricity and steam. An alternative to pyrolysis is high temperature and pressure supercritical water decomposition (hydrothermal monophasic oxidation).

### Pyrolysis-Pyrolysis is a process of thermo-chemically decomposition of organic materials by heat in the absence of oxygen which produces various hydrocarbon gases.[18] During pyrolysis, the molecules of object are subjected to very high temperatures leading to very high vibrations. Therefore, every molecule in the object is stretched and shaken to an extent that molecules starts breaking down. The rate of pyrolysis increases with temperature. In industrial applications, temperatures are above 430 °C (800 °F).[19] Fast pyrolysis produces liquid fuel for feedstocks like wood. Slow pyrolysis produces gases and solid charcoal.[20] Pyrolysis hold promise for conversion of waste biomass into useful liquid fuel. Pyrolysis of waste plastics can produce millions of litres of fuel. Solid products of this process contain metals, glass, sand and pyrolysis coke which cannot be converted to gas in the process.

### Resource recovery-Resource recovery is the systematic diversion of waste, which was intended for disposal, for a specific next use.[21] It is the processing of recyclables to extract or recover materials and resources, or convert to energy.[22] These activities are performed at a resource recovery facility.[22] Resource recovery is not only environmentally important, but it is also cost effective.[23] It decreases the amount of waste for disposal, saves space in landfills, and conserves natural resources.[23]

Resource recovery (as opposed to waste management) uses LCA (life cycle analysis) attempts to offer alternatives to waste management. For mixed MSW (Municipal Solid Waste) a number of broad studies have indicated that administration, source separation and collection followed by reuse and recycling of the non-organic fraction and energy and compost/fertilizer production of the organic material via anaerobic digestion to be the favoured path.

As an example of how resource recycling can be beneficial, many of the items thrown away contain precious metals which can be recycled to create a profit, such as the components in circuit boards. Other industries can also benefit from resource recycling with the wood chippings in pallets and other packaging materials being passed onto sectors such as the horticultural profession. In this instance, workers can use the recycled chips to create paths, walkways, or arena surfaces.

### Sustainability-The management of waste is a key component in a business' ability to maintaining ISO14001 accreditation. Companies are encouraged to improve their environmental efficiencies each year by eliminating waste through resource recovery practices, which are sustainability-related activities. One way to do this is by shifting away from waste management to resource recovery practices like recycling materials such as glass, food scraps, paper and cardboard, plastic bottles and metal. a lot of conferences will discuss this topic as the international Conference on Green Urabnism which will be held in Italy From 12–14 October 2016

Avoidance and reduction methods -An important method of waste management is the prevention of waste material being created, also known as waste reduction. Methods of avoidance include reuse of second-hand products, repairing broken items instead of buying new, designing products to be refillable or reusable (such as cotton instead of plastic shopping bags), encouraging consumers to avoid using disposable products (such as disposable cutlery), removing any food/liquid remains from cans and packaging,[24] and designing products that use less material to achieve the same purpose (for example, lightweighting of beverage cans).